FIS DR cutover procedure

- After the business decision is made to cut over to the DR link, perform the following steps

- Log into the Denver Palo Alto firewall

- Navigate to Network -> Virtual Routers -> vrouter-1

- Click on Static Routes

- Enter FIS in the search box to see all 8 static routes for FIS

- Check the box next to each route then click “Delete”

name-FIS1, dest-156.55.112.0/21, next-hop ip address- 172.29.98.43

name-FIS2, dest-170.132.0.0/16, next-hop ip address- 172.29.98.43

name-FIS3, dest-10.118.123.0/24, next-hop ip address- 172.29.98.43

name-FIS4, dest-12.195.6.192/26, next-hop ip address- 172.29.98.43

name-FIS5, dest-12.195.6.192/26, next-hop ip address- 172.29.98.43

name-FIS6, dest-156.55.82.0/24, next-hop ip address- 172.29.98.43

name-FIS7, dest-10.96.37.0/27, next-hop ip address- 172.29.98.43

name-FIS8, dest-156.55.49.0/24, next-hop ip address- 172.29.98.43

- This will remove all FIS routes from the Denver PANs.

- Commit policy on the Denver Palo Alto firewall

- Add routes for the FIS networks in Plano

- In Plano add the following commands on vdcthr1-inet and vdcthr2-inet

ip prefix-list ospf-static-redist seq 200 permit 156.55.112.0/21

ip prefix-list ospf-static-redist seq 205 permit 172.132.0.0/16

ip prefix-list ospf-static-redist seq 210 permit 10.118.123.0/24

ip prefix-list ospf-static-redist seq 215 permit 12.195.6.192/26

ip prefix-list ospf-static-redist seq 220 permit 168.162.0.0/16

ip prefix-list ospf-static-redist seq 225 permit 156.55.82.0/24

ip prefix-list ospf-static-redist seq 230 permit 10.96.37.0/27

ip prefix-list ospf-static-redist seq 235 permit 156.55.49.0/24

- This will enable the Plano switches to start advertising the FIS routes out of Thornton.

- copy run start on both switches

- verify that the preferred route to Plano for the FIS networks are showing up in the vdcden1-inet and vdcden2-inet

- Have testers attempt to connect to FIS and verify traffic is being seen on the Plano Palo Alto firewalls.

FIS Fail back procedure

- Verify with the business that they are ready to cut back to sending FIS traffic out of Denver

- Log into vdcthr1-inet and vdcthr2-inet

- Remove the FIS routes from the prefix list added in step 10 above

no ip prefix-list ospf-static-redist seq 200 permit 156.55.112.0/21

no ip prefix-list ospf-static-redist seq 205 permit 172.132.0.0/16

no ip prefix-list ospf-static-redist seq 210 permit 10.118.123.0/24

no ip prefix-list ospf-static-redist seq 215 permit 12.195.6.192.26

no ip prefix-list ospf-static-redist seq 220 permit 168.162.0.0/16

no ip prefix-list ospf-static-redist seq 225 permit 156.55.82.0/24

no ip prefix-list ospf-static-redist seq 230 permit 10.96.37.0/27

no ip prefix-list ospf-static-redist seq 235 permit 156.55.49.0/24

- Copy run start on both switches

- Log back into the Denver Palo Alto firewalls and recreate the 30 FIS routes that were removed in step 6 above.  See screenshot for values to enter.

- Commit policy on Denver Palo Alto firewalls.

- Verify routes are correct on the network and verify traffic is hitting the Denver Palo Alto firewalls.