Proposed steps to create lacp in DR environment between  Palo Alto firewalls and cisco switches.

- Smart hands will install additional link between port 1/47 on thr1wan9k and thr2wan9k switches.

2.04 RU 29 PT 47  2.05 RU 29 PT 47

- Smart hands will install uplinks from port 1/22 FWDR5 to port 1/53

2.04 PT 21 --> 2.04 RU 29 PT 51
2.04 PT 22--> 2.05 RU 24 PT 51

2.05 PT 21 --> 2.05 RU 29 PT 53
2.05 PT 22 --> 2.04 RU 24 PT 53

- Take backups of existing configuration on Panorama and Cisco Nexus switches.

- Implement cisco nexus side configuration changes first.

thr1wan9k

interface port-channel51

description aws_to_drfws

switchport

switchport mode access

switchport access vlan 3811

spanning-tree port type edge

interface port-channel47

description trunk_to_thr2wan9k

switchport

switchport mode access

switchport access vlan 3812 (Same or different vlan)

interface eth1/51

description aws_to_drfw6_p21

channel-group 51 mode active

interface eth1/53

description aws_to_drfw5_p21

channel-group 51 mode active

interface eth147

description trunk_to_thr2wan9k

channel-group 47 mode active

interface eth1/48

description trunk_to_thr2wan9k

channel-group 47 mode active

thr2wan9k

interface port-channel51

description aws_to_drfws

switchport

switchport mode access

switchport access vlan 3811

spanning-tree port type edge

interface eth1/51

description aws_to_drfw6_p21

channel-group 51 mode active

interface eth1/53

description aws_to_drfw5_p21

channel-group 51 mode active

interface port-channel47

description trunk_to_thr2wan9k

switchport mode trunk

switchport trunk allowed vlan 3811

interface eth1/51

channel-group 47 mode active

interface eth1/53

channel-group 47 mode active

- Panorama configuration will be implemented using follow panorama template:

delete template lacp-template config vsys vsys1 zone aws_private network layer3 [ethernet1/1]

delete template lacp-template config vsys vsys1 zone SwiftDR-Untrust network layer3 [ethernet1/22]

delete template lacp-template network interface ethernet ethernet1/21

delete template lacp-template network interface ethernet ethernet1/22

set template lacp-template config network interface aggregate-ethernet ae1 layer3 lacp enable yes

set template lacp-template config network interface aggregate-ethernet ae1 lacp mode active

set template lacp-template config network interface aggregate-ethernet ae1 lacp system-priority 100

set template lacp-template config network interface aggregate-ethernet ae1 layer3 ip 172.24.0.241/29

set template lacp-template network interface ethernet ethernet1/21 aggregate-group ae1

set template lacp-template network interface ethernet ethernet1/22 aggregate-group ae1

set template lacp-template config network virtual-router vrouter-1 interface [ ae1 ]

set template lacp-template config vsys vsys1 zone aws_private network layer3 [ ae1 ]

Logical Layer 3 VLAN interface vlan 3211 and assign an IP address: Switch> enable Switch# config term Enter configuration commands, one per line. End with CNTL/Z. Switch(config)# vlan 3811 Switch(config)# interface vlan 32811 Switch(config-if)# ip address

Switch(config-if)# no shutdown Switch(config-if)# end